| Scope | This document describes pre-blanching of chicken livers in their primary packaging |
| --- | --- |
| Objectives | Accelerate defrosting of frozen chicken livers |
| Responsibility | Quality controller Blancher Operator General worker Process controller Process Engineer |
| Document references | Name Doc No Defrosted Liver – Blancher input |

Blanching

- Refer to SIC form ……and follow steps

- Feed water to above the water level mark engraved in the blancher

- Heat water above 95 degrees Celsius

- Set the time to 7 Minutes

- Decant two bins (approximate 90kg) of defrosted chicken livers into the blancher

- After 3 minutes, stir gently so as not to damage the liver and allow evenly distribution of heat and free residues

- Decant the content into water at room temperature to reduce the heat

Cooling the product

- Ensure that the cooling machine is fed with cold water before blanching the liver

- Receive the blanched liver from the blancher – water temperature MUST be below 50 degrees.

- Lift the cooler twice to release the residues off the cooled liver

- Transfer to the small white bins

- Weigh the content and record in the form

- QC periodically checks the temperature of the cooler and instructs the draining of half of the water if 50 and above.

- Feed additional cold water to continue the process.

- Transfer weighed, blanched chicken livers to the batching station

- Weigh and record the remains after batching into white cups.

Standard Operating Procedure: Blancher

Note: BEWARE OF HOT WATER

Blanching Process

Timing: 6 minutes on hot water side – 1 minute on cold water side

- Retrieve Raw Liver Containers:

- Retrieve two white containers of raw liver from the defrosting and drainage station, ensuring each container weighs approximately 45kg.

- Temperature Settings:

- Ensure the hot water side of the blancher is set to a temperature between 95°C - 100°C.

- Verify that the cold-water side temperature is between 20°C - 50°C using a thermometer.

- Liver Inspection:

- Visually inspect the liver in the white buckets. If excessive blood is visible, redrain the liver before proceeding.

- Loading the Blancher:

- Combine the contents of the two white containers, totaling 90kg – 95kg of drained raw liver, and place them in the hot water side of the blancher.

- Blanching Process:

- Activate the stopwatch.

- Every 30 seconds, lift the draining bin in the hot water to remove any liver sediments.

- Activate the turning arm only once in either hot or cold water, and only to remove or dump the liver.

- Transfer to Cold Water:

- After 6 minutes, dump the liver from the hot water side into the cold-water side.

- After 30 seconds in the cold water, lift the draining bin to remove any liver sediments.

- Final Dumping:

- After 1 minute in the cold water, dump the liver into small white containers (3 containers per load).

- Place the blanched livers at the filling station weigh point.

Replacement of Hot and Cold Water

Note: Only clean one blancher at a time

- Frequency of Replacement:

- Replace the hot and cold water every 20 loads of 90kg – 95kg.

- Draining the Blancher:

- CAUTION: Beware of hot water on the hot water side of the blancher and when draining.

- Drain the water from the blancher by opening the stop valves and directing the water into the drains.

- Cleaning the Blancher:

- Clean all sediments from the hot and cold-water sides of the blancher using a water hose.

- Keep the drain valves open during the cleaning process.

- Refilling the Blancher:

- Refill the blancher with hose water on both the hot and cold-water sides.

- Ensure the filling level is set to 16 holes from the top of the draining bin in the blancher.

- Verify that the hot water side temperature is between 95°C - 100°C before loading a new batch.

End of Night Shift Procedure

- Draining the Blancher:

- CAUTION: Beware of hot water on the hot water side of the blancher and when draining.

- Drain the water from the blancher by opening the stop valves and directing the water into the drains.

- Cleaning the Blancher:

- Clean all sediments from the hot and cold-water sides of the blancher using a water hose.

- Keep the drain valves open during the cleaning process.

- Refilling the Blancher:

- Refill the blancher with hose water on both the hot and cold-water sides.

- Ensure the filling level is set to 16 holes from the top of the draining bin in the blancher.

- Final Check:

- Do not turn on the heating element.